AIO9 System Hardware Block Diagram Document

Detailed Subsystem Block Diagrams & Inter-Board Signals: ESP32 Board, GPRS Modem Board & Nuvoton UI Board

This document provides the complete hardware block diagrams and signal interface maps for the three primary circuit boards in the AIO9 system architecture: the ESP32 Core Controller Board, the SIMCom GPRS Modem Board, and the Nuvoton UI Board.

# 1. Top-Level Multi-Board Architecture & Flow Diagram

The AIO9 architecture decouples main processing, cellular communication, and screen/keypad user interaction across three dedicated hardware boards:

| ■ ESP32 Core Board (Master Controller) • Dual-Core Xtensa LX6 (240MHz) • Core 0: Network & GPRS Tasks • Core 1: Scheduler & Sensors • ULP Coprocessor (Pulse Counter) • RTC Slow RAM (State Memory) • SPIFFS (16MB Flash) & SD Card • DS1307 Real Time Clock (I2C) • HDC1080 / BME280 Sensors | ■ GPRS Modem Board (Cellular Subsystem) • SIMCom A7672S / A7672M6 • LTE Cat-1 4G + 2G GSM Fallback • Direct Layer-4 TCP Engine • Multi-Carrier SIM Slot (Airtel/BSNL) • RF Front-End & Antenna • 4.0V Power Regulator • 2000uF Peak Current Capacitors | ■ Nuvoton UI Board (HMI Subsystem) • Nuvoton MG51 MCU (8051-Core) • 16x2 Alphanumeric LCD Display • 6-Key Tactile Keypad Scan • UART Display Protocol Engine • HD44780 Command Parser • EXT INT Keypress Generator • Continuous 5V Sleep Power |
| --- | --- | --- |

| ◄── UART1 (GPIO 14 RX / GPIO 4 TX @ 115200 Baud) ──► | ESP32 ◄──► Nuvoton UI Data Line | Bidirectional screen drawing & remapped keypress stream |
| --- | --- | --- |
| ◄── UART2 (GPIO 16 RX / GPIO 17 TX @ 115200 Baud) ──► | ESP32 ◄──► GPRS Modem Line | AT command interface & Direct TCP socket payload stream |
| ──► EXT INT (GPIO 27 ext0 LOW) & PWR (GPIO 32) ──► | ESP32 ◄──► Nuvoton Sleep & Wake | Nuvoton pulls INT LOW on keypress to wake ESP32 from Deep Sleep |

# 2. ESP32 Core Controller Board Block Diagram

| ■ Dual-Core Processing Engine (ESP32 Microcontroller) • Core 0: Dedicated GPRS stack, TCP sockets, AT parsing • Core 1: Scheduler loop, sensor sampling, UI rendering • FreeRTOS preemptive multitasking & semaphore locks | ■ ULP Coprocessor & RTC RAM (Low Power Subsystem) • ULP Coprocessor running 1ms sampling loop during sleep • Pulse counting for Rain Gauge & Wind Anemometer • RTC Slow Memory preserving state across deep sleep | ■ Storage & Filesystem (Non-Volatile Memory) • 16MB Flash memory with SPIFFS filesystem • Stores cur_file, unsent.txt queue, APN configs • SPI SD Card slot for offline log backups & SD OTA |
| --- | --- | --- |
| ■ I2C Sensor Bus (GPIO 21/22) (Hardware Sensors) • DS1307 Real Time Clock (0x68) for system timestamping • HDC1080 / HDC2022 (0x40) Temperature & Humidity sensor • BME280 Barometric Pressure sensor (Sea level correction) | ■ Power Control & ADCs (Power Management) • 3.7V - 4.2V Li-ion Battery & Solar Input channels • Calibrated Battery Voltage sense via ADC pins • GPIO 32: Nuvoton 5V Power Gate (Held HIGH in sleep) • GPIO 26: GPRS Modem Power Enable MOSFET Gate | ■ External I/O Interfaces (Peripherals) • GPIO 27: ext0 External Interrupt input from Nuvoton • GPIO 14/4: UART1 serial interface to Nuvoton UI • GPIO 16/17: UART2 serial interface to SIMCom modem • GPIO 33: Hardware Reset line to SIMCom modem |

# 3. GPRS Modem Board Block Diagram

| ■ Cellular Core Engine (SIMCom Module) • SIMCom A7672S / A7670 / A7672M6 Chipset • LTE Cat-1 4G Network Engine with 2G GSM Fallback • 115200 Baud UART AT Command interface • PWRKEY & RESET Hardware control inputs | ■ Direct Layer-4 TCP Engine (Firmware Socket Stack) • Bypasses modem high-level HTTP parser defects • Layer-4 Sockets (AT+NETOPEN, AT+CIPOPEN, AT+CIPSEND) • Direct IP connection to server (No DNS delay) • Instant '>' buffer prompt response (< 5ms) |
| --- | --- |
| ■ SIM Interface & Discovery (Carrier Engine) • Dual/Single SIM Slot supporting 1.8V / 3.0V SIM cards • Auto Carrier Discovery via ICCID (899116 / 899145) • Supports Airtel M2M, Airtel Commercial, BSNL, Jio • Automatic APN selection (airteliot.com / bsnlnet) | ■ Power & RF Front-End (Hardware Power Rail) • Dedicated 4.0V Voltage Regulator rail • 2000uF Tantalum/Bulk capacitors for 2A peak GPRS bursts • Main Cellular Antenna connector & RF Matching Network • Signal quality RSSI / CSQ detection circuit |

# 4. Nuvoton UI Board Block Diagram

| ■ Microcontroller Core (Nuvoton MG51 MCU) • Nuvoton MG51 8051-core high-speed MCU • Dedicated firmware for LCD control & Keypad scanning • 115200 Baud UART Interface to ESP32 (GPIO 14/4) • Receives display update commands from ESP32 | ■ Alphanumeric LCD Module (Display Engine) • 16x2 Character Alphanumeric LCD display • HD44780 Command Parser emulation • Supports display clear (0x01), cursor position (0xC0) • Display ON/OFF control (0x0C / 0x08) for power saving |
| --- | --- |
| ■ Tactile Matrix Keypad (User Inputs) • 6-Button Tactile Keypad (UP, DOWN, LEFT, RIGHT, SET, CLR) • Continuous key Matrix scanning & hardware debouncing • Remaps key output to AIO9 internal key codes • Transmits ASCII key codes over UART to ESP32 | ■ Power & Wakeup Control (Power Subsystem) • Powered continuously via 5V rail (ESP32 GPIO 32 HIGH) • Dedicated INT pin tied to ESP32 GPIO 27 (ext0) • Pulls INT LOW on any keypress to wake ESP32 from Deep Sleep • Allows zero keypress loss during deep sleep |

# 5. Signal & Interconnection Pinout Matrix

| Subsystem | Signal Name | ESP32 Pin | Target Board Pin | Type / Protocol | Description & Logic |
| --- | --- | --- | --- | --- | --- |
| Nuvoton UI | UI_TX | GPIO 4 | Nuvoton RX | UART Out (115200 Baud) | Transmits screen drawing & cursor commands from ESP32 to Nuvoton. |
| Nuvoton UI | UI_RX | GPIO 14 | Nuvoton TX | UART In (115200 Baud) | Receives remapped keypress byte stream from Nuvoton keypad. |
| Nuvoton UI | UI_INT | GPIO 27 | Nuvoton INT | Digital Input (ext0 LOW) | Keypress trigger line that wakes ESP32 from Deep Sleep mode. |
| Nuvoton UI | UI_PWR | GPIO 32 | Nuvoton 5V Gate | Digital Output (MOSFET) | Kept HIGH in sleep mode so Nuvoton remains powered for keypress wakeup. |
| GPRS Modem | GPRS_TX | GPIO 17 | SIMCom RXD | UART Out (115200 Baud) | Sends AT commands and raw Layer-4 TCP socket HTTP POST payloads. |
| GPRS Modem | GPRS_RX | GPIO 16 | SIMCom TXD | UART In (115200 Baud) | Receives modem responses, TCP connection status, and HTTP 200 OK. |
| GPRS Modem | GPRS_PWR | GPIO 26 / 5 | SIMCom PWRKEY | Digital Output | Power pulse control line to turn SIMCom modem ON/OFF. |
| GPRS Modem | GPRS_RST | GPIO 33 / 4 | SIMCom RESET | Digital Output | Hardware emergency reset trigger for modem recovery. |
| Sensors & RTC | I2C_SDA | GPIO 21 | RTC / Sensors SDA | I2C Data (400kHz) | Shared I2C data bus for DS1307 RTC, HDC1080/2022, and BME280. |
| Sensors & RTC | I2C_SCL | GPIO 22 | RTC / Sensors SCL | I2C Clock (400kHz) | Shared I2C clock bus for all onboard environmental sensors. |
| Rain / Wind | RAIN_IN | GPIO (ULP) | Tipping Bucket | Digital Pulse Input | Rain gauge tipping bucket pulses counted by ULP coprocessor. |
| Rain / Wind | WIND_IN | GPIO (ULP) | Anemometer | Digital Pulse Input | Wind speed reed switch pulses counted by ULP coprocessor. |
| Power Sense | BAT_SENSE | GPIO 34-39 | Battery Divider | Analog ADC Input | Calibrated battery voltage divider measurement input. |