ISO 9001:2015 QUALITY MANAGEMENT SYSTEM | STANDARD OPERATING PROCEDURE

Automated Factory Test Jig System & Quality Control SOP

| Document Ref: | SOP-QC-ESP32-AIO-2026-v6 | ISO Clause Alignment: | 7.1.5 & 8.5.1 |
| --- | --- | --- | --- |
| Target System: | Spatika ESP32 Telemetry Board | Revision / Ver: | v6.32 / QC-v6.13 |
| Classification: | Controlled Audit Manual | Effective Date: | September 2026 |

1. Purpose & ISO 9001 Compliance Mandate

This Standard Operating Procedure (SOP) defines the operational workflow, hardware test jig setup, software verification routines, and automated quality audit logging for assembled Spatika ESP32 Weather Telemetry Circuit Boards (TRG, TWS, TWS-RF, and TWSRP variants).

This quality system strictly complies with ISO 9001:2015 Requirements:

Clause 7.1.5 (Monitoring and Measuring Resources): Guarantees that hardware verification equipment produces valid, repeatable, and verifiable test results for all sensors, power management circuits, and GPRS modems.

Clause 8.5.1 (Control of Production and Service Provision): Mandates controlled factory conditions, automated Pass/Fail quality gates, and 100% cloud-traceable serial number logging prior to field deployment.

2. System Architecture & Test Jig Hardware Setup

The Quality Control Test Jig consists of three integrated operational layers: Hardware Fixture, Firmware Diagnostic Engine, and WebSerial Browser Dashboard.

+---------------------------------------------------------------------------------+ ¦ SPATIKA TEST JIG SYSTEM ¦ ¦ ¦ ¦ +------------------------+ WebSerial (USB) +-----------------------+ ¦ ¦ ¦ Web Dashboard ¦<========================>¦ ESP32 Target Board ¦ ¦ ¦ ¦ (factory_tool.html) ¦ ¦ (Running qc_test.ino) ¦ ¦ ¦ +-----------+------------+ +-----------+-----------+ ¦ ¦ ¦ ¦ ¦ ¦ ¦ Automated Cloud Log API ¦ Hardware ¦ ¦ v v Tests ¦ ¦ +------------------------+ +-----------------------+ ¦ ¦ ¦ Google Sheets ISO Log ¦ ¦ Sensors, Modem, RTC, ¦ ¦ ¦ (Central Quality Audit)¦ ¦ Nuvoton UI, Power ADC ¦ ¦ ¦ +------------------------+ +-----------------------+ ¦ +---------------------------------------------------------------------------------+

2.1 Hardware Interface & Pin Assignment

The target ESP32 board is placed on the test jig fixture via pogo-pin connectors. The test jig verifies the following pin assignments:

| Subsystem / Peripheral | ESP32 Pin | Signal Type | Test Jig Verification Criteria |
| --- | --- | --- | --- |
| GPRS Board Power Gate | GPIO 26 | Digital Output (PMOS) | Active-HIGH power control verification |
| Nuvoton UI Power Gate | GPIO 32 | Digital Output (PMOS) | Active-HIGH power control verification |
| Keypad SET Interrupt | GPIO 27 | Digital Input (PULLUP) | Hardware interrupt trigger on keypress |
| Nuvoton UART Interface | TX 14 / RX 4 | UART Serial1 (9600) | ASCII character loopback & display render |
| Modem UART Interface | TX 17 / RX 16 | UART Serial2 (115200) | SIMCom A7672S AT command response |
| Rain Sensor Input | GPIO 34 | Analog / Interrupt | Pulse count interrupt (50ms debounce window) |
| Wind Speed Anemometer | GPIO 35 | Analog / Interrupt | Pulse count interrupt (15ms debounce window) |
| Wind Direction ADC | GPIO 39 | ADC Channel (Raw) | Calibrated ADC voltage read (0 - 3480 mV) |
| Solar Panel Voltage | GPIO 25 | ADC Channel (Divider) | Resistor divider voltage read (R1=620K, R2=100K) |
| Battery Voltage (Li-ion) | GPIO 33 | ADC Channel (eFuse) | eFuse-calibrated ADC voltage read (3.4V - 4.2V) |
| Rain Pulse Simulator | GPIO 2 | Digital Output | Internal loopback pulse generator (GPIO 2 -> GPIO 34) |

3. Automated QC Test Firmware Specification (qc_test.ino)

Upon powering on the test jig, the dedicated Quality Control firmware (qc_test.ino) executes a deterministic state machine validating every onboard peripheral.

| Test Order | Peripheral Module | Diagnostic Method & Threshold | Acceptance Criteria |
| --- | --- | --- | --- |
| 1 | System Core & MAC | Reads ESP32 Chip ID, Revision, MAC Address | Valid MAC address string generated |
| 2 | SPIFFS Filesystem | Mounts partition, formats if unformatted, writes /qc.txt | File write/read loopback verified |
| 3 | SD Card Socket | Initializes SPI SD (CS GPIO 5 or 13), checks /firmware.bin | SD Card mounted; File I/O verified |
| 4 | RTC Clock (I2C) | Queries DS1307/DS3231 @ 0x68. Clears CH halt bit if set | Seconds register advances over 2500ms |
| 5 | Temp/Humidity Sensor | Scans I2C at 0x40 (HDC1080/2022) or 0x76/0x77 (BME280) | Valid temperature & humidity reads |
| 6 | Wi-Fi Transceiver | Scans STA mode Wi-Fi networks and measures RSSI | Scan returns >= 1 access point |
| 7 | GPRS Modem & SIM | Powers GPIO 26, sends AT, checks CPIN, CCID, & CREG | SIM Ready + Registered via CREG (2G/4G) |
| 8 | Power Rails ADC | Reads battery (GPIO 33) and solar (GPIO 25) ADC | Battery voltage > 3.40V |
| 9 | Nuvoton UI & Keypad | Interactive Keypad test: CLEAR, UP, DOWN, LEFT, RIGHT, SET | All 6 keys pressed & confirmed by operator |
| 10 | Rain Gauge Interrupt | Operator manually tips tipping bucket (or simulated pulse) | Pulse count matches tipped count |
| 11 | Deep Sleep & RTC | Puts MCU to Deep Sleep; verifies timer wakeup & persistence | MCU wakes up cleanly via timer reset |

4. Operational Workflow & Factory Dashboard (factory_tool.html)

4.1 Step-by-Step Operator Workflow

Step 1 (Launch Jig Dashboard): Double-click launch_mac.command (macOS) or launch_windows.bat (Windows). This launches the local HTTP server on port 8000 and automatically opens Chrome/Edge at http://localhost:8000/factory_tool.html.

Step 2 (Connect Target Board): Mount assembled ESP32 target board onto test jig fixture. Click 'CONNECT DEVICE' on the WebSerial dashboard and select the USB Serial port.

Step 3 (Automated QC Execution): Click 'FLASH QC FIRMWARE'. The tool flashes qc_test_8mb.bin to the target board. The automated test sequence executes, rendering live green/red test cards for each hardware module.

Step 4 (Interactive Keypad & Rain Gauge Verification): The operator presses front-panel keypad keys (CLEAR, UP, DOWN, LEFT, RIGHT, SET) as prompted on the LCD. Next, the operator manually tips the rain gauge bucket. The jig counts and displays exact tip tallies.

Step 5 (Automated Production Flashing & Cloud Sync): Upon QC PASS, the system automatically flashes the target Production Application firmware (e.g. v6.32) via SD-OTA or WebSerial, and automatically posts the complete audit record to Google Sheets.

4.2 Automated SD-to-ESP32 Production Firmware Flash (OTA)

Inside qc_test.ino, the function flashProductionFirmwareFromSD() automatically updates the ESP32 to production firmware directly from the onboard SD card (/firmware.bin) upon test approval:

// Automatically flashes production firmware from SD card upon QC PASS bool flashProductionFirmwareFromSD(bool rebootAfterFlash) { File firmware = SD.open("/firmware.bin", FILE_READ); int firstByte = firmware.read(); if (firstByte != 0xE9) { // Verify ESP32 binary header magic byte Serial.println("[QC_OTA] FAIL: CORRUPT BINARY"); return false; } firmware.seek(0); Update.onProgress(otaProgressCallback); if (Update.begin(firmware.size(), U_FLASH)) { size_t written = Update.writeStream(firmware); if (written == firmware.size() && Update.end(true)) { Serial.println("[QC_OTA] COMPLETE: Production FW Written Successfully"); ESP.restart(); // Boot into production application } } }

5. Quality Record Traceability & Cloud Audit Logging

To satisfy ISO 9001 Clause 8.5.1 traceability requirements, every tested board generates an immutable JSON quality record uploaded directly to a centralized Google Sheets Quality Log.

5.1 Quality Audit JSON Payload Structure

{ "timestamp": "2026-09-10T12:15:00Z", "station_id": "001802", "esp32_mac": "44:1D:64:CC:9C:88", "qc_ver": "6.13-QC", "prod_fw_ver": "6.32", "overall_verdict": "PASS", "spiffs_status": "PASS", "sd_card_status": "PASS", "rtc_status": "PASS", "sensor_type": "HDC1080", "temperature_c": 29.34, "humidity_pct": 60.05, "wifi_aps_found": 4, "gprs_modem_status": "PASS", "sim_ccid": "89914526034065788200", "signal_rssi_dbm": -51, "battery_voltage": 4.04, "keypad_status": "PASS", "rain_gauge_tips": 10, "operator_id": "QC_TECH_02" }

6. Automated Build & Packaging Verification (build_jig.sh)

The automated shell script build_jig.sh guarantees that factory test jig binaries are compiled with strict version alignment:

Automated Compilation: Uses arduino-cli to compile 8MB and 16MB QC test binaries (qc_test_8mb.bin) and production binaries (production_8mb.bin).

Version Enforcement: Extracts FIRMWARE_VERSION from user_config.h and verifies that the embedded binary strings match expected release tags before packaging.

HTML & Static Code Verification: Executes verify_html.py and find_unbalanced_diff.py to inspect factory_tool.html for syntax errors, missing tags, or unbalanced brackets prior to deployment.

7. ISO Quality Control Acceptance Criteria & Checklist

| Check Item | ISO Standard Requirement | Verification Pass Threshold | Verdict (Pass/Fail) |
| --- | --- | --- | --- |
| CPU & MAC ID | Traceability (ISO 8.5.1) | Unique ESP32 MAC address queried & logged | PASS [ ] FAIL [ ] |
| Filesystem R/W | Data Integrity (ISO 7.1.5) | SPIFFS & SD read/write loopback test clean | PASS [ ] FAIL [ ] |
| Clock Accuracy | Calibration (ISO 7.1.5) | RTC crystal ticks over 2.5s; CH bit cleared | PASS [ ] FAIL [ ] |
| Sensor Calibration | Measurement Validity | Temperature within 15-45°C; Humidity 20-90% | PASS [ ] FAIL [ ] |
| Cellular Connectivity | Functional Verification | SIM CCID parsed; CREG registered; CSQ >= -95dBm | PASS [ ] FAIL [ ] |
| Power Protection | Safety & Reliability | Battery ADC > 3.40V; 2A RF surge brownout check | PASS [ ] FAIL [ ] |
| UI Keypad Operations | User Interface Controls | All 6 keys (CLEAR, UP, DOWN, LEFT, RIGHT, SET) ACKed | PASS [ ] FAIL [ ] |
| Rain Pulse Counter | Transducer Verification | 10 manual tipping bucket pulses counted exactly | PASS [ ] FAIL [ ] |
| Production OTA | Controlled Deployment | Production FW flashed cleanly; Magic 0xE9 verified | PASS [ ] FAIL [ ] |
| Cloud Log Entry | Quality Audit Record | JSON record published to Google Sheets Quality Database | PASS [ ] FAIL [ ] |

ISO 9001 QUALITY AUDIT SIGN-OFF & APPROVAL

| Quality Assurance Engineer: | ___________________________ | Date: | ____ / ____ / 2026 |
| --- | --- | --- | --- |
| Lead ISO Auditor: | ___________________________ | Status: | APPROVED [ ] REJECTED [ ] |