Technical Architecture Document:
Nuvoton UART UI Subsystem

Document Reference: ARCH-NUVOTON-UI-2026-v6  |  Target Platform: ESP32 AIO Weather Station  |  Date: September 2026

1. Executive Overview & System Context

Prior to firmware version v5.91, the All-In-One (AIO) ESP32 weather telemetry platform relied exclusively on a direct I2C-based LCD (HD44780 controller driven via PCF8574 I/O expander) coupled with a 3x4 GPIO matrix keypad (Matrix UI, configured via #define USE_NUVOTON_UI 0).

While cost-effective, the Matrix UI architecture introduced two critical system bottlenecks as sensor suite and logging capabilities expanded:

I2C Bus Contention: Screen redrawing heavily saturated the single I2C bus shared with the Real-Time Clock (DS3231) and high-precision environmental sensors (HDC1080 / BMP280), creating mutex lock contention under heavy sampling loads.

GPIO Pin Exhaustion & Floating Noise: Scanning the 3x4 keypad matrix required 5 dedicated GPIO pins (GPIO 4, 12, 13, 14, 15). During sleep cycles or when LCD power was toggled off, floating input lines frequently triggered electrical noise false keypresses (ghost keypresses).

To eliminate these bottlenecks in v6.xx, support was added for an external Nuvoton Microcontroller UI Offloader (Nuvoton UI, configured via #define USE_NUVOTON_UI 1). The Nuvoton MCU manages keypad scanning, display refresh, and front-panel LED indicators natively, communicating with the main ESP32 over a lightweight 2-wire high-speed UART interface (Serial1 @ 9600 baud).

2. Hardware Architecture Comparison

The shift from an I2C-driven display and GPIO-scanned matrix keypad to a UART offloader fundamentally simplifies the ESP32 hardware interconnect footprint:

| Subsystem Component | Legacy Matrix UI (USE_NUVOTON_UI = 0) | Modern Nuvoton UI (USE_NUVOTON_UI = 1) |
| --- | --- | --- |
| Primary Display Interface | I2C Bus (Wire) @ Address 0x27 / 0x3F | UART Hardware Serial (Serial1) |
| Physical ESP32 Pins | SDA (GPIO 21), SCL (GPIO 22) | TX (GPIO 14), RX (GPIO 4) |
| UART Baud Rate / Config | N/A | 9600 baud, 8N1 |
| Keypad Input Method | Direct Row/Column Scan via Keypad.h | Offloaded to Nuvoton; Keycodes sent via UART |
| Keypad GPIO Footprint | 5 Pins: Rows {4, 12}, Cols {13, 14, 15} | 0 extra pins (Reuses UART RX/TX) |
| Display Mutex Overhead | Requires i2cMutex lock for every write | Zero I2C bus lock overhead |
| Sleep Power Isolation | Sets keypad GPIOs to INPUT_PULLUP | Drives GPIO 32 LOW + calls Serial1.end() |
| Ghost Press Guard | I2C ACK ping at 0x27/0x3F before key process | 90ms UART refractory filter + FIFO Draining |

3. Firmware Abstraction & Protocol Mapping

3.1 Drop-In Class Emulation (NuvotonLCD)

To avoid refactoring thousands of lines of UI rendering logic across lcdkeypad.ino, gprs_health.ino, and webServer.ino, a drop-in C++ class NuvotonLCD was implemented in lcdkeypad.ino. It exposes the exact same method signatures as LiquidCrystal_I2C, translating function calls into UART byte commands:

#if USE_NUVOTON_UI == 1 class NuvotonLCD { public: void init(); void clear(); void setCursor(uint8_t col, uint8_t row); void print(const char* str); void print(String str); void print(char c); void print(int num); void print(float val, int decimals = 2); void blink(); void noBlink(); void cursor(); void noCursor(); void backlight(); void noBacklight(); }; #endif

3.2 Protocol Byte Mapping Specification

The NuvotonLCD class serializes high-level display directives into raw byte sequences sent over Serial1:

| Method API | Sent UART Bytes / Sequence | Operation Description |
| --- | --- | --- |
| clear() | 0x01 | Wipes display buffer and moves cursor to (0,0) |
| setCursor(col, row) | (uint8_t)(row * 64 + col + 128) | Sets HD44780 DDRAM address pointer |
| print(str) | Serial1.write(str, len) | Streams raw ASCII bytes to Nuvoton display buffer |
| blink() | 0x0F | Enables cursor blinking |
| noBlink() | 0x0C | Disables cursor blinking |
| backlight() | 0x0C | Enables LCD LED backlight |
| noBacklight() | 0x08 | Disables LCD LED backlight |

4. Keypad Input Processing & Signal Sanitization

4.1 Key Code Translation Table

The Nuvoton MCU scans the front-panel keys and transmits ASCII characters '1' through '6' over Serial1. The ESP32 maps these inputs to standard internal UI actions via translate_nuvoton_key():

char translate_nuvoton_key(char n_key) { switch(n_key) { case '1': return '1'; // Nuvoton CLR -> Action: CLEAR case '2': return '4'; // Nuvoton LEFT -> Action: LEFT case '3': return '2'; // Nuvoton UP -> Action: UP case '4': return '5'; // Nuvoton DOWN -> Action: DOWN case '5': return '6'; // Nuvoton RIGHT -> Action: RIGHT case '6': return '3'; // Nuvoton SET -> Action: SET (also GPIO27 wakeup) default: return '0'; } }

4.2 Signal Sanitization & Multi-Tap Rejection

To filter switch contact bounce and duplicate UART frames during long key presses, a two-tier filtering mechanism is active:

90ms Refractory Lockout: Filters out duplicate UART frames received within 90ms of a valid keypress.

UART FIFO Draining: Immediately following a valid key detection, the ESP32 drains any leftover bytes in the UART receiver buffer (while (Serial1.available()) Serial1.read();).

5. Power Management, Deep Sleep & Current Leakage

5.1 Power Gating via GPIO 32

Physical power to the Nuvoton MCU is controlled by a MOSFET gate tied to GPIO 32. Driving GPIO 32 HIGH powers on the Nuvoton MCU, while driving it LOW cuts physical power during Deep Sleep.

5.2 Deep Sleep Leakage Prevention (Serial1.end())

If an active hardware serial peripheral remains initialized when the ESP32 enters Deep Sleep, internal pull-up/pull-down resistors on RX (GPIO 4) and TX (GPIO 14) draw up to 1.2mA of residual leakage current.

To guarantee sub-100µA Deep Sleep power consumption, Serial1.end() is called before sleep entry to completely release the pin states:

// Executed before Deep Sleep entry in lcdkeypad.ino digitalWrite(32, LOW); // Cut VCC power rail Serial1.end(); // Terminate Serial1 peripheral & release GPIO 4 & 14

6. System Performance & Build Configuration

By offloading screen rendering to UART Serial1, display updates no longer require i2cMutex locks. The I2C bus remains 100% dedicated to the Real-Time Clock (DS3231) and environmental sensors (HDC1080/BMP280), reducing mutex wait times from ~45ms per frame down to 0ms.

The UI subsystem is selected at compile-time via user_config.h:

// user_config.h // 1: Enable Nuvoton UART LCD/Keypad Subsystem (TX=GPIO14, RX=GPIO4) // 0: Enable Legacy I2C LCD (0x27/0x3F) + GPIO 3x4 Matrix Keypad #define USE_NUVOTON_UI 1